SECTION 900

TRAFFIC ACCOMMODATION AND CONTROL

SECTION 901—MAINTENANCE AND PROTECTION OF TRAFFIC DURING CONSTRUCTION

901.1  DESCRIPTION—This work is the furnishing, installing, maintaining, and relocating of traffic control devices. This work may include flagging and pilot car operation for guidance of traffic through the temporary traffic control zone and dust control. All work shall be done as specified in these Specifications, the Publication 212, Publication 213, and the MUTCD, the Special Provisions, Standard Drawings, the approved Traffic Control Plans (TCP), and as directed.

901.2  MATERIAL—Furnish material and traffic control devices necessary for maintenance and protection of traffic, and conforming to the TCP, Publication 212, Publication 213, and the MUTCD, and as follows:

- Temporary Barrier—Section 627.2

- Temporary Concrete Barrier, Structure Mounted—Section 643.2

- Temporary Concrete Median Barrier, Structure Mounted—Section 643.2

- Temporary Concrete Glare Screen Median Barrier, Structure Mounted—Section 643.2

- Temporary Impact Attenuating Devices—Section 696.2

- Reset Temporary Barrier—Section 628.2

- Temporary Concrete Barrier, Structure Mounted, Reset—Section 644.2

- Temporary Concrete Median Barrier, Structure Mounted, Reset—Section 644.2

- Temporary Concrete Glare Screen Median Barrier, Structure Mounted, Reset—Section 644.2

- Reset Temporary Impact Attenuating Devices—Section 697.2

- Painting Traffic Lines and Markings—Section 962.2

- Asphalt Tack Coat—Section 460.2

- Superpave Asphalt Mixture Design, Asphalt Wearing Course 4.75 mm, SRL L—Section 413.2 and as follows: PG 64S-22, <0.3 million ESALs, or as indicated or directed.

- Shadow Vehicle—A truck equipped with a flashing or revolving yellow light and as specified in Publication 212, Publication 213, and the MUTCD.

- Truck Mounted Impact Attenuator and/or Arrow Panel—Installed on the shadow vehicle, as required in Publication 212, Publication 213, and the MUTCD.

- Adhesive—Section 966.2(c)

- Temporary Traffic Control Signals—Section 958.2

- Temporary Traffic Control Signals, Reset—Section 958.2

- Temporary Timing Adjustments to Existing Permanent Traffic Signal Controller—Section 958.2

Provide traffic line paint and glass beads or pavement marking tape according to the requirements of Section 901.3(k). All temporary traffic control devices must be listed in Bulletin 15.  Temporary Type III barricades must comply with Standard Drawings or be listed in Bulletin 15.  Temporary portable ‘H’ base and ‘X’ base sign posts must comply with the Standard Drawings or be listed in Bulletin 15.

Certification Form CS-4171 is not required for temporary traffic control devices.  However, temporary traffic control devices permanently incorporated as part of the project require Form CS-4171, unless otherwise specified in the contract.

901.3  CONSTRUCTION—

SHOWN HERE IS ONLY THE REVISION TO THE TEMPORARY TRAFFIC CONTROL SIGNAL SECTION. OTHER MATERIAL WILL REMAIN BUT IT IS NOT SHOWN FOR THE PURPOSES OF THE REVIEW.

(x)  Temporary Traffic Control Signals.  In accordance with Section 958.3:

For new temporary traffic control signals, this work is the furnishing and installation of complete and operational temporary traffic control signal systems for stationary operations; work includes installing, maintaining, relocating, resetting, and removing these temporary traffic control signals.

For existing permanent traffic control signals, this work is the temporary modification and restoration of existing permanent traffic control signals to address congestion resulting from traffic volume shifts due to construction-related, detoured traffic.

901.4  MEASUREMENT AND PAYMENT—

(a)  Maintenance and Protection of Traffic During Construction.  Lump Sum

The Department will measure and pay for this item in a proportionate manner based on current estimates.

With the exception of the separate pay items specified in Section 901.4(b), if an item or device is required for maintenance and protection of traffic, the cost of the item or device is incidental to Item 0901-0001.

(b)  Separate Pay Items.  The Department will separately measure and pay for the following items or devices, when indicated or required for maintenance and protection of traffic during construction:

Arrow Panel—Each or Day

Includes maintenance, fuel, and other operating costs for the payment indicated.

If required by a figure in Publication 213 this work is incidental to Item 0901-0001

Tubular Markers—Each

- Changeable Message Sign—Each

Includes maintenance, fuel, telecommunication service and other operating costs, relocating to other locations, and programming of messages as directed by the District Traffic Engineer or authorized Representative.

- Temporary Traffic Control Signals—Section 958.4 (Pay Items are 0958 items)

- Temporary Traffic Control Signals, Reset—Section 958.4 (Pay Items are 0958 items)

- Temporary Timing Adjustments to Existing Permanent Traffic Signal Controller—Section 958.4 (Pay Items are 0958 items)

- Floodlights—Each or Day

- Temporary Barrier—Section 627.4

- Temporary Concrete Barrier, Structure Mounted—Section 643.4

- Temporary Concrete Median Barrier, Structure Mounted—Section 643.4

- Temporary Concrete Glare Screen Median Barrier, Structure Mounted—Section 643.4

- Reset Temporary Barrier—Section 628.4

- Temporary Concrete Barrier, Structure Mounted, Reset—Section 644.4

- Temporary Concrete Median Barrier, Structure Mounted, Reset—Section 644.4

- Temporary Concrete Glare Screen Median Barrier, Structure Mounted, Reset—Section 644.4

- Painting Traffic Lines and Markings—Section 962.4

- Pavement Marking Removal—Section 963.4

Temporary Nonplowable Raised Pavement Markers—Each

- Temporary Asphalt Rumble Strips—Square Yard

- Temporary Impact Attenuating Devices—Section 696.4

- Reset Temporary Impact Attenuating Devices—Section 697.4

- Standard Pavement Markings, Paint & Beads—Linear Foot

- Standard Pavement Markings, Tape—Linear Foot

- Additional Warning Lights—Day

For the type indicated.

Based on an accumulation of 24 hours when in place for one unit.

- Additional Traffic Control Signs—Square Foot

Based on the minimum size sign for the necessary application as defined in Publication 212, Publication 213, and the MUTCD, unless otherwise directed.

- Temporary Highway Lighting—Lump Sum or Each

As indicated.

- Shadow Vehicle—Each or Day

Includes maintenance, fuel, relocations, and operating costs for the duration of the project.

If required by a figure in Publication 213 this work is incidental to Item 0901-0001

- Speed Display Signs—Each or Day

Includes maintenance, fuel and other operating costs, relocations, and programming.

- Roll-Up Vinyl Sign—Square Foot

Based on the square foot of existing sign.

Includes, mounting hardware and 3inches of material around perimeter of vinyl sign cover.