# SECTION 935—POST MOUNTED SIGNS, TYPE F

935.1  DESCRIPTION—This work is the furnishing and installation of signs of the type indicated on existing posts or poles installed for other purposes including the vertical portion of any traffic signal support, luminaire support, or sign structure support.

935.2  MATERIAL—As shown on the Standard Drawing for the corresponding type post and as follows:

- Flat Sheet Signs – Section 1103.4

- Brackets and Bars (Supports) – Section 1103.12

- Extruded Aluminum Channel Signs – Section 1103. 2

- Flat Sheet Aluminum Signs and Stiffeners – Section 1103. 3

- Galvanized Steel Hex Head Bolts, Nuts, Lock-Washers; Aluminum Post Clips; Lag Screws; Rivets; Anti-Theft Sign Hardware (System A) – Section 1103.11

- Internally Illuminated Signs—Section 1103.03(g)

- School Zone Speed Limit Flashing Warning Signs – Sections 951.2, 952.2, 954.2, 955.2, and 1103.03(h) or otherwise indicated on the approved plans.

- Flashing Warning Signs - Sections 950.2, 951.2, 952.2, 954.2, 955.2, and 1103.3(i), or otherwise indicated on the approved plans.

- LED Blank-Out Signs - Sections 950.2, 951.2, 952.2, 954.2, and 1103.3(j), or otherwise indicated on the approved plans.

- LED Border Lit Signs – Sections 950.2, 951.2, 952.2, 953.2, 954.2, and 1103.3(k), or otherwise indicated on the approved plans.

- Permanent Speed Display Signs - Sections 950.2, 951.2, 952.2, 954.2, 956.2(c)3, and 1103.3(m), or otherwise indicated on the approved plans.

- Temporary Speed Display Signs - Sections 950.2, 951.2, 952.2, 954.2, 956.2(c)3, and 1103.3(n), or otherwise indicated on the approved plans.

- Rectangular Rapid Flashing Beacon - Sections 950.2, 951.2, 952.2, 953.2, 954.2, 956.2(d), and 1103.3(o), or otherwise indicated on the approved plans.

935.3  CONSTRUCTION—Attach signs to supports for post mounted signs, Type A, B, C, D, or E or traffic signal support or luminaire supports, as shown on the Standards Drawing.

Remove existing signs only if indicated.  Signs that are removed become property of PennDOT unless otherwise indicated.

.

When mounting as a secondary sign below another sign, install sign at a minimum height of 5 feet on conventional highways in rural area; at 7 feet on conventional highways, on business, commercial, and residential streets where parking or pedestrian movements are likely to occur, or where other obstructions exist; and at 5 feet on expressways and freeways. Height is measured as the difference in elevation between near roadway edge surface and sign bottom.

Erect signs in a level position, set correctly on sign brackets, and attach firmly with the necessary mounting hardware as shown in the Standard Drawings or otherwise indicated on the approved plans.

Erect signs by a method that does not twist, bend, or otherwise deform the signs. Repair or replace damaged signs.

Permanently scribe 1-inch numerals on the back of sign indicating the month and year of installation.

Provide Internally Illuminated Signs from a manufacturer listed in Bulletin 15. Install as indicated in 1103.03(g) or otherwise indicated on the approved plans.

Provide School Zone Speed Limit Flashing Warning Signs from a manufacturer listed in Bulletin 15. Install as indicated in Sections 950.3, 951.3, 952.3, 954.3, 955.3, and 1103.3(h) or otherwise indicated on the approved plans.

Provide Flashing Warning Signs from a manufacturer listed in Bulletin 15. Install as indicated in Sections 950.3, 951.3, 952.3, 954.3, 955.3, and 1103.3(i) or otherwise indicated on the approved plans.

Provide LED Blank-Out Signs from a manufacturer listed in Bulletin 15. Install as indicated in Sections 950.3, 951.3, 952.3, 953.3, 954.3, 955.3, and 1103.3(j) or otherwise indicated on the approved plans.

Provide LED Border Lit Sign in compliance with MUTCD and department standards and provisions. Install as indicated in Sections 950.3, 951.3, 952.3, 953.3, 954.3, 955.3, and 1103.3(k) or otherwise indicated on the approved plans.

Provide Permanent Speed Display Signs from a manufacturer listed in Bulletin 15. Install as indicated and in Sections 950.3, 951.3, 952.3, 954.3, 955.3, 956.3(a)3, and 1103.3(m) or otherwise indicated on the approved plans.

Provide Temporary Speed Display Signs from a manufacturer listed in Bulletin 15. Install as indicated in Sections 950.3, 951.3, 952.3, 954.3, 955.3, 956.3(a)3, and 1103.3(n) or otherwise indicated on the approved plans.

Provide Rectangular Rapid Flashing Beacon Assembly from a manufacturer listed in Bulletin 15. Install as indicated in Sections 950.3, 951.3, 952.3, 953.3, 954.3, 956.3(b), and 1103.3(o) or otherwise indicated on the approved plans.

935.4  MEASUREMENT AND PAYMENT

(a)  Post Mounted Signs. Square Foot of sign.

The unit price includes removal of existing signs and mounting hardware.

(b)  Internally Illuminated Sign.  Each.

Includes sign assembly and all necessary connections, wiring, weather-tight wire entrance junction box, photoelectric cells, swing brackets, strapping, bolts, washers, and associated hardware to make the signs fully operational.

(c) School Zone Speed Limit Flashing Warning Signs. Each.

Includes sign assembly and Light Emitting Diode (LED) indications, controller assembly, time clock, solar components (if applicable), wiring, and associated hardware to make the sign fully operational.  Electrical service, trenching, and conduit is incidental to this sign.

(d) Flashing Warning Signs. Each.

Includes sign assembly and Light Emitting Diode (LED) indications, controller assembly, time clock, solar components (if applicable), wiring, and associated hardware to make the sign fully operational. Electrical service (if applicable), trenching, and conduit is incidental to the sign.

(e) LED Blank-Out Sign.  Each.

Includes sign assembly and all necessary connections, wiring, photoelectric cells,  brackets, strapping, bolts, washers, and associated hardware to make the signs fully operational.  Electrical service, trenching, and conduit is incidental to the sign.

(f) LED Border Lit Sign.  Each.

Includes sign assembly with Light Emitting Diode (LED) border, controller assembly, time clock, wiring, solar components (if applicable), detector (if applicable), controller communications (if applicable), and associated hardware to make the sign fully operational.  Electrical service, trenching, and conduit is incidental to the sign.

(g) Permanent Speed Display Sign.  Each.

Includes sign assembly with dynamic display panel, speed measuring detector, controller assembly with programmable schedule option, light sensor, “YOUR SPEED” sign (if applicable), solar components (if applicable), controller communications (if applicable), wiring, and associated hardware to make the sign fully operational.  Electrical service, trenching, and conduit is incidental to the sign.

(h) Temporary Speed Display Sign.  Each.

Includes sign assembly with dynamic display panel, speed measuring detector, controller assembly with programmable schedule option, light sensor, “YOUR SPEED” sign (if applicable), solar components (if applicable), controller communications (if applicable), wiring, and associated hardware to make the sign fully operational.  Electrical service, trenching, and conduit is incidental to the sign.

(i) Rectangular Rapid Flashing Beacon Assembly.  Each.

Includes crossing sign(s), controller assembly, wireless communication system, rapid flashing beacon unit, pedestrian confirmation light, push button (if applicable), push button sign (if applicable), pedestrian detection (if applicable), solar components (if applicable), wiring, and associated hardware to make the sign assembly fully operational.  Electrical service, trenching, and conduit is incidental to the sign.