SECTION 936—STRUCTURE MOUNTED SIGNS

936.1  DESCRIPTION—This work is the furnishing and installation of signs of the type indicated, mounted on the horizontal portion of overhead structures including span wires and mast arms.

936.2  MATERIAL—As shown on the Standard Drawings and as follows:

- Extruded Channel Signs and Miscellaneous Material—Section 930.2(a)

- Flat Sheet Aluminum Signs with Stiffeners and Miscellaneous Material—Section 930.2(b)

- Flat Sheet Signs—Section 1103.4

- Other Miscellaneous Material:

Angles—Section 1103.12(g)

Angles—Section 1105.2(a)2, galvanized as specified in Section 1105.2(s).

- Internally Illuminated Signs—Section 1103.3(g)

- School Zone Speed Limit Flashing Warning Signs – Sections 951.2, 952.2, 954.2, 955.2, and 1103.3(h) or otherwise indicated on the approved plans.

- Flashing Warning Signs - Sections 950.2, 951.2, 952.2, 954.2, 955.2, and 1103.3(i), or otherwise indicated on the approved plans.

- LED Blank-Out Signs - Sections 950.2, 951.2, 952.2, 954.2, and 1103.3(j), or otherwise indicated on the approved plans.

- LED Border Lit Signs – Sections 950.2, 951.2, 952.2, 953.2, 954.2, and 1103.3(k), or otherwise indicated on the approved plans.

- LED Red Signal Ahead Signs - Sections 950.2, 951.2, 952.2, 953.2, 954.2, and 1103.3(l), or otherwise indicated on the approved plans.

- Rectangular Rapid Flashing Beacon - Sections 950.2, 951.2, 952.2, 953.2, 954.2, 956.2(d), and 1103.3(o), or otherwise indicated on the approved plans.

936.3  CONSTRUCTION—Attach signs to structure  as shown on the Standard Drawings, as specified in Section 930.3(h) and as follows:

Remove existing signs only if indicated.  Signs that are removed become property of PennDOT unless otherwise indicated.

Submit to the Department all street name sign layout sheets for review and approval before fabrication.

Erect signs level, position correctly on sign brackets, and attach firmly with the necessary mounting hardware as shown in the Standard Drawings.

Erect signs by a method that does not twist, bend, or otherwise deform the signs. Repair or replace damaged signs.

Verify that the lowest point of any sign over the roadway is not less than 17 feet above the roadway surface unless otherwise specified on the approved plans.

Permanently scribe 1-inch numerals on the back of sign indicating the month and year of installation.

Provide Internally Illuminated Signs from a manufacturer listed in Bulletin 15. Install as indicated in 1103.03(g) or otherwise indicated on the approved plans.

Provide School Zone Speed Limit Flashing Warning Signs from a manufacturer listed in Bulletin 15. Install as indicated in Sections 950.3, 951.3, 952.3, 954.3, 955.3, and 1103.3(h) or otherwise indicated on the approved plans.

Provide Flashing Warning Signs from a manufacturer listed in Bulletin 15. Install as indicated in Sections 950.3, 951.3, 952.3, 954.3, 955.3, and 1103.3(i) or otherwise indicated on the approved plans.

Provide LED Blank-Out Signs from a manufacturer listed in Bulletin 15. Install as indicated in Sections 950.3, 951.3, 952.3, 953.3, 954.3, 955.3, and 1103.3(j) or otherwise indicated on the approved plans.

Provide LED Border Lit Sign in compliance with MUTCD and department standards and provisions. Install as indicated in Sections 950.3, 951.3, 952.3, 953.3, 954.3, 955.3, and 1103.3(k) or otherwise indicated on the approved plans.

Provide LED Red Signal Ahead Sign from a manufacturer listed in Bulletin 15. Install as indicated in Sections 950.3, 951.3, 952.3, 953.3, 954.3, 955.3, and 1103.3(l) or otherwise indicated on the approved plans.

Provide Rectangular Rapid Flashing Beacon Assembly from a manufacturer listed in Bulletin 15. Install as indicated in Sections 950.3, 951.3, 952.3, 953.3, 954.3, 956.3(b), and 1103.3(o) or otherwise indicated on the approved plans.

936.4  MEASUREMENT AND PAYMENT—

(a)  Structure Mounted Signs. Square Foot of sign.

The unit price includes removal of existing signs and mounting hardware.

(b)  Internally Illuminated Sign.  Each.

Includes sign assembly and all necessary connections, wiring, weather-tight wire entrance junction box, photoelectric cells, swing brackets, strapping, bolts, washers, and associated hardware to make the signs fully operational.

(c) School Zone Speed Limit Flashing Warning Signs. Each.

Includes sign assembly, Light Emitting Diode (LED) indications, controller assembly, time clock, solar components (if applicable), wiring, and associated hardware to make the sign fully operational. Electrical service, trenching, and conduit is incidental to the sign.

(d) Flashing Warning Signs. Each.

Includes sign assembly, Light Emitting Diode (LED) indications, controller assembly, time clock, solar components (if applicable), wiring, and associated hardware to make the sign fully operational. Electrical service, trenching, and conduit is incidental to the sign.

(e) LED Blank-Out Sign.  Each.

Includes all Blank-out sign assembly and necessary connections, wiring, photoelectric cells,  brackets, strapping, bolts, washers, and associated hardware to make the signs fully operational. Electrical service, trenching, and conduit is incidental to the sign.

(f) LED Border Lit Sign.  Each.

Includes sign assembly with Light Emitting Diode (LED) border, controller assembly, time clock, wiring, solar components (if applicable), detector (if applicable), controller communications (if applicable), and associated hardware to make the sign fully operational. Electrical service, trenching, and conduit is incidental to the sign.

(g) LED Red Signal Ahead Sign.  Each.

Includes sign assembly and all necessary connections, wiring, photoelectric cells, brackets, strapping, bolts, washers, and associated hardware to make the signs fully operational. Signal retiming, electrical service, trenching, and conduit is incidental to the sign.

(h) Rectangular Rapid Flashing Beacon Assembly.  Each.

Includes crossing sign(s), controller assembly, wireless communication system, rapid flashing beacon unit, pedestrian confirmation light, push button (if applicable), push button sign (if applicable), passive pedestrian detection (if applicable), solar components (if applicable), wiring, and associated hardware to make the sign assembly fully operational.  Electrical service, trenching, and conduit is incidental to the sign.