Overall Pub 408 Traffic Signal Specification Changes:

- Created several New Traffic Signal Sections:

  - 950-Traffic Signals-General.

  - 957-Adaptive Signal Systems.

  - 958-Temporary Traffic Control Signals.

  - 976-Remove Existing Structure Mounted Signs.

- Moved existing ‘Section 1104 -Traffic Signals’ information (General & Material) into corresponding §95X Sections; 95X Sections include other revisions as well:

  - 950.2 /950.3 Traffic Signals-General (now includes 1104.01).

  - 951.2 Supports (now includes 1104.02).

  - 952.2 Controller Assembly (now includes 1104.03).

  - 953.2 Systems (now includes 1104.04).

  - 954.2 Electrical (now includes 1104.05).

  - 955.2 Signal Heads (now includes 1104.06).

  - 956.2 Detectors (now includes 1104.07).

  - Existing Section 1104 will completely go away, as 1104 information is now combined into corresponding §95X sections.

- Moved existing ‘Section 1124-Temporary Traffic Control Signals’ information into New 958 Section:

  - Existing Section 1124 will completely go away, as 1124 information is now included in this New 958 section.

- In total, the following Pub 408 Sections will be affected with revisions: 901, 935, 936, 950, 951, 952, 953, 954, 955, 956, 957, 958, 975, 976, 1103, 1104, and 1124 – see below.

- New Master Items have been created as part of these specification revisions, including some Master Item replacements – see below.

- Various other corresponding PennDOT Publications will also be revised so that the above 408 specification revisions and publication’s guidance & requirements align.

Specific Section Revisions:

Section 901 -- MP&T During Construction

- Revised the Section 1124 reference links to Section 958 reference links for Temporary Traffic Signals and Temporary Traffic Signals, Reset.

- Added bullet point for Temporary Timing Adjustments to Existing Permanent Traffic Signal Controller, as included in Section 958.

- Measurement and Payment- The existing 0901-xxxx Master Items for Temporary Traffic Control Signals will be replaced with New 0958-xxxx Master Items to correspond with the newly created 958 Section for Temporary Traffic Control Signals. References back to Section 958.

Section 950 -- Traffic Signals-General

- Includes moved, existing Section 1104.01 general requirements information that did not fit well into the 95X sections.

- Written as a whole, complete specification (Description/Material/Construction/Measure & Payment).

- Ties together the ‘overall’ signal picture with all of its signal components, and references back to appropriate 95X links.

- Organizes and places all the various Traffic Signal Testing & Compliance requirements into one specification sub-section.

  - Adds testing language for ‘Emergency Generator Power/Uninterruptible Power Supply (UPS)

- Outline adds new topic information:

  - Traffic Signals Components.

  - Integration with Regional TMCs

  - Signals Maintenance/Repair Responsibilities During Construction.

  - TSAMS Updates.

  - Warranties (5 Yr. Warranty).

  - Unforeseen Traffic Signal Work.

- Measurement & Payment, adds language and Master Items for the following:

  - TSAMS Updates - Labor incidental to appropriate 95X section.

  - Traffic Signal Intersection Removal – Each.

  - Unforeseen Traffic Signal Work – Dollar.

Section 935—Post Mounted Signs, Type F

- Description, adds language for traffic signal, luminaire, and sign structure supports.

- Material, adds language and Master Items for the following:

  - Internally Illuminated Signs

  - School Zone Speed Limit Flashing Warning Lights

  - Flashing Warning Signs

  - LED Blank-Out Signs

  - LED Border Lit Signs

  - Permanent Speed Display Signs

  - Temporary Speed Display Signs

  - Rectangular Rapid Flashing Beacon

- Construction, adds new language for:

  - Setting and repairing signs

  - Standards for

    - Internally Illuminated Signs

    - School Zone Speed Limit Flashing Warning Signs

    - Flashing Warning Signs

    - LED Blank-Out Signs

    - LED Border Lit Sign

    - Permanent and Temporary Speed Display Signs

    - Rectangular Rapid Flashing Beacon Assembly

- Measurement and Payment, adds language and Master Items for the following:

  - Post Mounted Signs-SQ FT.

  - Internally Illuminated Sign- Each.

  - School Zone Speed Limit Flashing Warning Signs-Each.

  - Flashing Warning Signs-Each.

  - LED Blank-Out Sign-Each.

  - LED Border Lit Sign-Each.

  - Permanent Speed Display Sign-Each.

  - Temporary Speed Display Sign-Each.

  - Rectangular Rapid Flashing Beacon Assembly-Each.

Section 936—Structure Mounted Signs

- Material, adds language and Master Items for the following:

  - LED Border Lit Signs

  - LED Red Signal Ahead Signs

  - Rectangular Rapid Flashing Beacon

- Construction, adds new language and information for:

  - LED Blank-Out Signs

  - LED Border Lit Sign

  - LED Red Signal Ahead Sign

  - Rectangular Rapid Flashing Beacon

- Measurement and payment, adds language or Master Items for the following:

  - Internally Illuminated Sign-Each.

  - School Zone Speed Limit Flashing Warning Signs-Each.

  - Flashing Warning Signs-Each.

  - LED Blank-Out Sign-Each.

  - LED Border Lit Sign-Each.

  - LED Red Signal Ahead Sign-Each.

  - Rectangular Rapid Flashing Beacon Assembly-Each.

Section 950—Traffic Signals -General

- Construction, adds language about:

  - Removed traffic signal equipment

  - Modification, alteration, removal, and/or storage of traffic signal material

  - Independent testing for controller bench test

  - Emergency generator power/ uninterruptible power supply (UPS) testing

  - Timing setting adjustments

  - Traffic signal operation training/technical support

  - Integration with department regional traffic management centers

  - Signal maintenance/repair responsibilities during construction

  - Department’s Traffic Signal Asset Management System (TSAMS) Updates

  - Warranties and Guarantees

  - Unforeseen traffic signal work

- Measurement and Payment, adds language for:

  - TSAMS Updates

  - Traffic signal intersection removal

  - Unforeseen traffic signal work

Section 951 -- Supports

- Measurement & Payment, adds language and Master Items for the following:

  - Span Wire Replacement – Each.

  - Span Tether Wire Replacement – Each.

  - Span Sag Adjustment (Retrofit of Signals) – Each.

  - Tether Wire Sag Adjustment (Retrofit of Signals) – Each.

  - Painting of Existing Traffic Signal Supports – Each.

Section 952 -- Controller Assembly

- Revised the GPS Time Clock

- Added Auxiliary Cabinet with and without Fan

- Remove the Type 170 Controller Assembly

- Added the Advanced Transportation Controller (ATC) Unit.

  - Added item for ATC Spare Parts

- Added language to require cabinet risers to type I cabinets

- Revised internal LED lighting, fans, and surge protection of the cabinet assemblies

- Added Controller modification (retiming, controller upgrade, and general) to the section

- Revised and reorganized the wording and item for Time Clock, GPS.

- Measurement & Payment, adds language and Master Items for the following:

  - Included Advanced Transportation Controller Unit with the Controller Assembly

  - Controller Modification, Retiming - Each

  - Controller Modification, Controller Upgrade - Each

  - Controller Modification - Each

  - Auxiliary Cabinet with Fan - Each

  - Auxiliary Cabinet without Fan - Each

  - Time Glock, GPS -Each

Section 953 –Traffic Signal Systems and Communications

- Material, adds information for:

  - Controller Assembly

  - Electrical Distribution

  - Signal Heads

  - Wireless broadband communication

  - Restore underground electrical and fiber optic facilities

  - Managed network switch

  - Layer 3 network switch

  - Advanced railroad interconnect

    - Management information bases

    - Software requirements

    - Hardware requirements

    - Controller requirements

    - Controller cabinet

    - Railroad preemption interface

  - Networking of school zone flashers

  - Ethernet bridge

- Construction, adds information for:

  - Wireless broadband communication

  - Managed network switch

  - Layer 3 network switch

  - Unified command and control integration

  - Signal timing adjustments

  - Advanced railroad interconnect

  - Networking of school zone flashers

- Measurement and Payment, adds information for:

  - Wireless Broadband Communication-Each.

  - Restore Underground Electrical and Fiber Optic Facilities-Dollar.

  - Managed Network Switch-Each.

  - Layer 3 Network- Each.

  - UCC Integration-Lump Sum.

  - Signal Timing Adjustments-Each.

  - Advanced Railroad Interconnect-Each.

  - Networking of School Zone Flashers-Lump Sum.

Section 954 -- Electrical

- Outline adds new topic information:

  - Wire removal clarification.

  - Ethernet cable material specifications

  - Use of automatic transfer relay switch for generator adaptor kits and UPS along with indicator lights.

  - Addition of True Line Interactive UPS systems.

  - Junction boxes added JB-30

  - Electrical service set-up requirement clarification.

  - Grade 3 or 4 stainless steel wool for conduits.

  - All conduits will be sealed 4” without damaging cable and inserting stainless steel to prevent rodent infestation.

  - Preserve foundation integrity when installing conduit into an existing foundation.

  - Item for Generator Adaptor Kit, including shop drawings and testing.

- Measurement & Payment, adds language and Master Items for the following:

  - Junction Box, JB-30 – Each.

  - Generator Adapter Kit Retrofit – Each.

  - Installation of Conduit to Existing Traffic Signal Support – Each.

Section -- 955 Heads

- Measurement & Payment, adds language and Master Items for the following:

  - Backplate Retrofit – Each.

Section 956 -- Detectors

- General Reorder of the section

- Provided specifications for Cat 5e/6 ethernet cable

- Loop Sensors

  - Added backer rod to loop sensors

  - Added a count component to the Loop Amplifier

- Video Detection – updated material and construction requirements

  - Updated video detection with increased technological specification requirements

  - Updated video detection to be IP addressable

  - Added a video detector with internal heater

  - Added a video detector that utilizes Thermal technology

  - Added a monitor and keyboard/mouse for video detection

- Added Omni-View Video Detection System that utilizes a 360-degree lens to track, detect, classify and count pedestrians, bicycles, and motor vehicles for the entire intersection.

- Added a video detection system that allows for the use of all the video detectors and systems described in the video detector section of 956.

- Radar Detector

  - Revised the requirements of the radar detector to match current special provisions.

  - Provided more guidance on the requirements for the detection zone.

  - Reorganized the radar detection system to match current special provisions.

- Radar and Video Detection System

  - Utilizes all the video and radar detectors and systems to provide a combination detection system.

- Revised the description of the material and construction for pedestrian push buttons, accessible pedestrian push signals, and preemption system to match current special provisions.

  - The section now requires push buttons require latching isolator card in the controller cabinet

  - The section details the specifications for APS and provides the operating procedure, with examples, on how the APS should function.

- Closed Circuit Television Camera for signalized intersections was added to this section to make an item specific for traffic signal installations.

- Measurement & Payment, adds language and Master Items for the following:

- Loop Amplifier, X Channel Shelf/Rack Mounted with timer, count output

- Radar Detection System

- Radar and Video Detection System

- Video Detector

- Video Detector with Internal Heater

- Thermal Video Detector

- Video Detection System

- Omni-View Video Detection System

- Monitor and Keyboard for Video Detection

- Closed Circuit Television Camera for Signalized Intersection

Section 957 -- Adaptive Signals

- New section that discusses Advanced Traffic Signal Technology and serves a home to future advance technology.

- Currently the section focuses on Adaptive Signal Systems (Hardware, Software, and Detection Systems).

  - Description, Material, Construction, and Measurement and Payment were laid out from scratch for all topics in this section.

  - Detection System leverages either/or quantity and multiple sections (910, 950, and 954) to provide guidance on material and construction.

Section 958 -- Temporary Signals

- Includes moved, existing Section 1124 requirements information, as well as other additional information. The New 958 section will keep temporary signals together with the other 95X specifications.

- Written as a whole, complete specification (Description/Material/Construction/Measure & Payment).

- Will include sub-sections for the following:

  - Fixed Supports Signals.

  - Pedestal-Mounted Portable Signals.

  - Trailer-Mounted Portable Signals.

  - Temporary Timing Adjustments to Existing Permanent Traffic Signal Controller.

- Outline adds new topic information:

  - Made Flash Period/Turn-on requirements, same as for permanent signals.

  - Two overhead-signals on portable-trailer signals.

    - Separate unique Item if requested (Optional).

  - WZ Luminaires on portable-trailer signals:

    - Separate unique Item if requested (Optional).

    - Not highway lighting, but the function of the WZ Luminaire is to illuminate the portable trailer signal and surrounding work area during periods of low visibility.

  - Equip unit with a generator connector for power loss conditions.

  - Adds the specific PATA figure references.

  - Emergency contact and response information requirements.

  - Timing adjustments requirements.

  - Complete all required temporary traffic control features (paint, barrier, signing, etc.) before the temporary traffic signal is turned on.

  - Repair surface areas disturbed by the installation of portable signals as directed and approved by the Representative.

  - When not in operation, signal heads shall be covered, turned, etc.

  - Install temporary traffic control signals on fixed supports to be in accordance with appropriate construction specifications located in the appropriate 95X sections.

  - New sub-section for Temporary Timing Adjustments to Existing Permanent Traffic Signal Controller. For existing permanent traffic signals, this work is the temporary modification and restoration of existing permanent traffic signals to address congestion resulting from traffic volume shifts due to construction-related detoured traffic.

- Measurement and Payment:

  - Eliminate 32 existing TTCS Master Items (0901 Items) and replace with 14 New Master Items (new 0958 Items).

    - Fixed-Supports - Each Complete WZ Setup.

    - Portable Pedestal-Mounted - Each Device.

    - Portable Trailer-Mounted - Each Device.

  - Eliminated combination ‘either/or’ Items for bid purposes:

    - Each item is a separate technology/method.

    - Still can bid Item A or Item B (example: pedestal item or trailer item; fixed support item or trailer item).

    - No single Item that states ‘either/or’ within the same item.

  - Added a new item for, Temporary Timing Adjustments to Existing Permanent Traffic Signal Controller.

    - Each Signalized Intersection

- Related Publication 213 Revisions to TTCS sections were made:

  - PATAs 701 thru 706.

  - Appendix D.

Section 975 -- Remove Post Mounted Signs, Type F

- No changes currently proposed

Section 976 -- Remove Existing Structure Mounted Signs

- Created a new section to address this type of sign removal with a pay Item.

Section 1103—Traffic Signing and Marking

- 1103.03 language added for:

  - School Zone Speed Limit Flashing Warning Sign; adding solar powered system

  - Flashing Warning Sign; adding solar powered system

  - Light Emitting Diode (LED) Blank-Out Signs;

  - LED Border Lit Signs

  - Light Emitting Diode (LED) Red Signal Ahead Signs

  - Permanent Speed Display Signs

  - Temporary Speed Display Signs

  - Rectangular Rapid Flashing Beacon Assembly

Section 1104 -- Traffic Signals

- Moved existing ‘Section 1104 -Traffic Signals’ information (General & Material) into corresponding §95X Sections.

- Existing Section 1104 will completely go away, as 1104 information is now combined into corresponding §95X sections.

Section 1124 -- Temporary Traffic Control Signals

- Moved existing ‘Section 1124-Temporary Traffic Control Signals’ information into New 958 Section.

- Existing Section 1124 will completely go away, as 1124 information is now included in this New 958 section.