# SECTION 935—POST MOUNTED SIGNS, TYPE F

935.1  DESCRIPTION—This work is the furnishing and installation of signs of the type indicated on existing posts or poles installed for other purposes including the vertical portion of any traffic signal support, luminaire support, or sign structure support.

935.2  MATERIAL—As shown on the Standard Drawing for the corresponding type post and as follows:

- Flat Sheet Signs – Section 1103.4

- Brackets and Bars (Supports) – Section 1103.12

- Extruded Aluminum Channel Signs – Section 1103. 2

- Flat Sheet Aluminum Signs and Stiffeners – Section 1103. 3

- Galvanized Steel Hex Head Bolts, Nuts, Lock-Washers; Aluminum Post Clips; Lag Screws; Rivets; Anti-Theft Sign Hardware (System A) – Section 1103.11

- Internally Illuminated Signs—Sections 950.2, 1103.03(g)

- School Zone Speed Limit Flashing Warning Signs – Sections 950.2, 951.2, 952.2, 954.2, 955.2, and 1103.03(h) or as indicated.

- Flashing Warning Signs - Sections 950.2, 951.2, 952.2, 954.2, 955.2, and 1103.3(i), or as indicated.

- Light Emitting Diode (LED) Blank-Out Signs - Sections 950.2, 951.2, 952.2, 954.2, and 1103.3(j), or as indicated.

- LED Border Lit Signs – Sections 950.2, 951.2, 952.2, 953.2, 954.2, and 1103.3(k), or as indicated.

- Permanent Speed Display Signs - Sections 950.2, 951.2, 952.2, 954.2, 956.2(c)3, and 1103.3(m), or as indicated.

- Temporary Speed Display Signs - Sections 950.2, 951.2, 952.2, 954.2, 956.2(c)3, and 1103.3(n), or as indicated.

- Rectangular Rapid Flashing Beacon - Sections 950.2, 951.2, 952.2, 953.2, 954.2, 956.2(d), and 1103.3(o), or as indicated.

935.3  CONSTRUCTION—Attach signs to supports for post mounted signs, Type A, B, C, D, or E or traffic signal support or luminaire supports, as shown on the Standard Drawing.

Remove existing signs if indicated.  Signs that are removed become property of PennDOT unless otherwise indicated.

When mounting as a secondary sign below another sign, install sign at a minimum height of 5 feet on conventional highways in rural area; at 7 feet on conventional highways, on business, commercial, and residential streets where parking or pedestrian movements are likely to occur, or where other obstructions exist; and at 5 feet on expressways and freeways. Height is measured as the difference in elevation between near roadway edge surface and sign bottom.

Erect signs in a level position, set correctly on sign brackets, and attach firmly with the necessary mounting hardware as shown on the Standard Drawings or as indicated .

Erect signs by a method that does not twist, bend, or otherwise deform the signs. Repair or replace damaged signs.

Permanently scribe 1 inch numerals on the back of sign indicating the month and year of installation.

Install Internally Illuminated Signs as specified in Sections 950.3 and 1103.03(g) or otherwise indicated.

Install School Zone Speed Limit Flashing Warning Signs as specified in Sections 950.3, 951.3, 952.3, 954.3, 955.3, and 1103.3(h) or as indicated.

Install Flashing Warning Signs as specified in Sections 950.3, 951.3, 952.3, 954.3, 955.3, and 1103.3(i) or as indicated.

Install LED Blank-Out Signs as specified in Sections 950.3, 951.3, 952.3, 953.3, 954.3, 955.3, and 1103.3(j) or as indicated.

Install LED Border Lit Signs conforming to the MUTCD and as shown in the standard drawings. Install as specified in Sections 950.3, 951.3, 952.3, 953.3, 954.3, 955.3, and 1103.3(k) or as indicated.

Install Permanent Speed Display Signs as specified in Sections 950.3, 951.3, 952.3, 954.3, 955.3, 956.3(a)3, and 1103.3(m) or as indicated.

Install Temporary Speed Display Signs as specified in Sections 950.3, 951.3, 952.3, 954.3, 955.3, 956.3(a)3, and 1103.3(n) or as indicated.

Install Rectangular Rapid Flashing Beacon Assembly as specified in Sections 950.3, 951.3, 952.3, 953.3, 954.3, 956.3(b), and 1103.3(o) or as indicated.

935.4  MEASUREMENT AND PAYMENT

(a)  Post Mounted Signs. Square Foot of sign.

The unit price includes removal of existing signs and mounting hardware.

(b)  Internally Illuminated Sign.  Each.

Includes sign assembly and all necessary connections, wiring, weather-tight wire entrance junction box, photoelectric cells, swing brackets, strapping, bolts, washers, and associated hardware to make the signs fully operational.

(c) School Zone Speed Limit Flashing Warning Signs. Each.

Includes sign assembly and LED indications, NEMA rated enclosure contained controller assembly, time clock, solar components (if applicable), wireless/cellular communication (if applicable), wiring, and associated hardware to make the sign fully operational.

(d) Flashing Warning Signs. Each.

Includes sign assembly and LED indications, controller assembly, time clock, solar components (if applicable), wiring, and associated hardware to make the sign fully operational.

(e) LED Blank-Out Sign.  Each.

Includes sign assembly and all necessary connections, wiring, photoelectric cells,  brackets, strapping, bolts, washers, and associated hardware to make the signs fully operational.

(f) LED Border Lit Sign.  Each.

Includes sign assembly with LED border, controller assembly, time clock, wiring, solar components (if applicable), detector (if applicable), controller communications (if applicable), and associated hardware to make the sign fully operational.

(g) Permanent Speed Display Sign.  Each.

Includes sign assembly with dynamic display panel, speed measuring detector, controller assembly with programmable schedule option, light sensor, “YOUR SPEED” sign (if applicable), solar components (if applicable), controller communications (if applicable), wiring, and associated hardware to make the sign fully operational.

(h) Temporary Speed Display Sign.  Each.

Includes sign assembly with dynamic display panel, speed measuring detector, controller assembly with programmable schedule option, light sensor, “YOUR SPEED” sign (if applicable), solar components (if applicable), controller communications (if applicable), wiring, and associated hardware to make the sign fully operational.

(i) Rectangular Rapid Flashing Beacon Assembly.  Each.

Includes  crossing sign(s), NEMA rated enclosure contained controller assembly, controller assembly, wireless communication system, rapid flashing beacon unit, pedestrian confirmation light, backplate (optional), push button (if applicable), push button sign (if applicable), passive pedestrian detection (if applicable), solar components (if applicable), wiring, and associated hardware to make the sign assembly fully operational.