SECTION 957—ADVANCED TRAFFIC SIGNAL TECHNOLOGY

957.1 DESCRIPTION—This work is the furnishing and installation of advanced traffic signal technology for the purpose of improving traffic operations and enabling communication between vehicles and infrastructure.

  - MATERIAL—

- General.

- Traffic Signals-General—Section 950.2

- Certification—Section 106.03(b)3

- Warranty and Support.

The supplier shall provide a 5-year technical support, hardware, and software warranty as applicable to the installed system. The supplier shall include a 5-year repair or replace guarantee including labor on defective hardware at no cost to the Department or traffic signal owner.  The supplier shall include five years of technical support available 24/7, 365 days of the year and five years of free software updates.

The system supplier shall maintain an ongoing program of technical support for the Advanced Traffic Signal Technology. This technical support shall be available via telephone or via personnel sent to the installation site upon placement of an order at the suppliers then current pricing and terms of sale for on-site technical support services.

The system supplier shall maintain an adequate inventory of parts to support maintenance and repair of the Advanced Traffic Signal Technology. Replacement parts shall be available for delivery within 30 days of an order at the supplier’s current pricing. The supplier shall guarantee support and replacement parts for up to 10 years for the Advanced Traffic Signal Technology. The supplier and/or contractor shall be responsible for the cost of replacement parts when the system is under the technical, hardware, and software support period.

The 5-year warranty commences at the later of the two dates; the hardware is energized or the date of software installation/Department and Municipality training.

- Adaptive Signal System

The Adaptive Signal System includes multiple intersections as indicated.

The Adaptive Signal System shall be fully compatible with the controller and vehicle detection system installed at all intersections within the system.

The Adaptive Signal System shall have a fallback state that allows coordination using a common cycle length for all intersections in the system. The Adaptive Signal System shall have a fallback state that allows individual intersections to operate in a vehicle-actuated, isolated mode in the event of failures of the adaptive software hardware, detectors, or communication.

The Adaptive Signal System shall be capable to optimize no less than every five minutes and shall provide updates to the adaptive software at a minimum of every four cycles to monitor performance and track the changes made to the splits, offsets, cycle lengths, and phasing sequences.

The Adaptive Signal System software shall be capable of remote monitoring and configuration via a web interface locally and over an Ethernet network on an authorized computer and shall be password protected.  The Adaptive Signal System shall be connected to the Commonwealth Network to allow PennDOT to monitor the operation of the system to ensure the operational goals are met. The Adaptive Signal System shall allow access by PennDOT and all parties authorized by the District Traffic Engineer.

The Adaptive Signal System shall collect and store locally real-time traffic data such as vehicle counts, stop delay and queue information per lane for a minimum of 4-weeks or as indicated. Real-time and historical statistical information shall be available in graphical and/or tabular form via a web browser. This data shall be available to all parties authorized by the District Traffic Engineer.

In the event of a detector, communications, or adaptive hardware or software failure, the Adaptive Signal System shall immediately trigger an alarm and at a minimum automatically send alphanumeric messages (short messaging service (SMS)/text messaging) to cellular telephones and email addresses to the specified parties as directed by the District Traffic Engineer.

In the event of detection failure, the Adaptive Signal System shall either revert to time of day, utilize other system detectors, vehicle recall settings, use historic system data from a similar time period to account for the approach or movement demand, or utilize adjacent or reliable downstream detection to account for vehicular demand.

The Adaptive Signal System shall not prevent access to the local signal controller database, monitoring or reporting functions by any installed signal management system.

The Adaptive Signal System shall have the ability to be operated and monitored from a TMC if applicable.

The Adaptive Signal System shall function with these minimum operation requirements:

- The Adaptive Signal system shall be capable of real-time corridor optimization, including dynamic adjustment of cycle length or timing pattern selection and coordination offsets with minimal transition period, the lesser of four cycles or five minutes

- The Adaptive Signal System shall handle real time incidents and traffic shifts by modifying the splits and offsets of the traffic pattern to adapt to increases and decreases in traffic flows on the facility it is managing. The Adaptive Signal System shall detect additional traffic and provide additional green time and adjust offsets as needed.

- Selection of timing pattern, cycle length, offset, and split calculation shall be based on achieving the objective of the adaptive system and be based on the detection evaluation of queues, wait times, and/or vehicle data.

- All timing pattern, cycle length, offset, and split thresholds shall be authorized by the District Traffic Engineer.

- Provide an Adaptive Signal System that can function in an actuated-coordinated mode or adaptive mode selectable by time of day and day of week.

- If pedestrian actuation and detection is applicable, the system shall accommodate pedestrian actuations.

- If preemption is applicable, the system shall allow all preemption phases to override the system, operate as per the approved preemption sequencing without impedance from the system, and resume adaptive operation upon completion of the preemption phases with minimal transition, lesser of four cycles of five minutes.

- If queue preemption is applicable, the system shall accommodate queue preemption.

- If railroad or transit preemption is applicable, the system shall accommodate railroad or transit preemption.

- If priority is applicable, the system shall allow all priority phases to override the system, operate as per the approved priority sequencing without impedance from the system, and resume adaptive operation upon completion of the priority with minimal transition, less than two cycles.

- The system shall be able to reservice side street phases while operating in its coordinated phase.

- When the system has a detection failure, it shall be capable of utilizing optimized signal splits based on historical split information or use other capable system detectors to maintain adaptive operations.

  - Hardware Adaptive Signal System

As specified in Section 957.2(b), 957.2(c), and as follows:

Provide all hardware necessary at each intersection for the installation of the Hardware Adaptive Traffic Signal System which shall include the following:

- Adaptive Traffic Signal Processor

- Equipment

- Spare equipment

- Web-based/adaptive system application/adaptive signal system interface and setup (if applicable)

(a) Adaptive Traffic Signal Processor.:

- The system shall provide equipment as necessary to communicate between the detection system(s) and the traffic adaptive components of the system.

- The system shall automatically send all necessary information to processors at neighboring intersections (peer to peer) in order to optimize the phasing sequence, duration, and initiation of movements along the corridor.

- The system shall be capable of passing all vehicle detection inputs through to the controller to allow the intersection to operate without adaptive control by manual command, time of day, or in the event of an adaptive processor failure.

- When the controller loses communication with the processor, the controller shall default to using existing time-of-day timings in conjunction with the vehicle detectors.  To accomplish this, all vehicle detection shall be passed through to the controller in the event of adaptive processor failure or a need to disable adaptive operation in the future.  The system shall not operate in “free mode” due to a loss of communication between the controller and the processor unless directed.

- The system shall have a “free mode” contact closure pin wired in the cabinet so if communication is lost between the hardware adaptive system and the controller, the intersection will revert to a prior time of day plan and generate an alarm, short messaging service (SMS) text message or email, to inform TMC/system operators.

(b) Equipment.

- Provide a minimum 10/100 plug-and-play Ethernet connection with static IP.

- For systems that connect detection into the adaptive signal system processors, include a lightning arrestor and fuse block to protect the power leads to the vehicle detectors.

- Allow power to be remotely cycled to the vehicle detectors.

- Provide one 7 inch LCD monitor and integrated USB keyboard/mouse for the entire system. The keyboard shall have key spacing (center to center) of 0.75 inches with a minimum 84 keys. The monitor and keyboard/mouse shall allow full monitoring and configuration of the adaptive system at each controller cabinet.  The monitor shall have a minimum resolution of 1024 x 768.  Upon final configuration of the adaptive system, place the monitor and keyboard/mouse in the controller cabinet at an intersection directed by the District Traffic Engineer.

- Provide a minimum 2 GB re-imaging flash drive for each intersection containing all the programming details at each intersection. Each flash drive is to be labeled and shall remain in the cabinet.

(c) Spare Equipment.

- Provide the number of spare systems as indicated. Each spare system shall include one processor, four vehicle detectors (or enough to fully operate one 4-way intersection) that match the detection system installed under the project, and one equipment panel (if required).  If no detection system is installed under the project, spare detection is not required. Deliver the spare system to the locations directed by the District Traffic Engineer.

  - Software Adaptive Signal System

As specified in Section 957.2(b), 957.2(c), and as follows:

Provide all necessary components for the installation of the Software Adaptive Traffic Signal System which shall include the following:

- Controller setup/programming/keys (at each intersection)

- Software license/setup by system or intersection

- Central System (if applicable) or licenses that connect and configure with already established Department Central System agreements.

- Web-based/adaptive system application/adaptive signal system interface and setup

- Off-site computer/keyboard/monitor and setup with the adaptive signal system manufacturer’s minimum system requirements if applicable as indicated.

The Software Adaptive Signal System features, programming, and/or operations shall not be limited by the brand of controller and vehicle detection system installed at all intersections within the system. The Software Adaptive System shall be 100% compatible with all controller equipment and vehicle detection installed within the system.

In the event of a minor communication failure or detection failure, the Software Adaptive Signal System shall utilize comparable historic data to develop splits, offsets, and cycle lengths or use other capable system detectors to maintain adaptive operations. The Adaptive Signal System shall immediately trigger an alarm and send a message, short messaging system (SMS) text message or email, to the specified parties as specified in Section 957.2(b).

In the event of a major communication or system failure, the Software Adaptive Signal System shall revert to time of day timing plans. The Adaptive Signal System shall immediately trigger an alarm and send a message, SMS text message or email, to the specified parties as specified in Section 957.2(b).

  - Intersection Adaptive System Vehicle Detection System

Each intersection shall be furnished with a vehicle detection system as specified in Sections 910.2, 950.2, 954.2, 956.2, and as follows:

The detection warranty shall be governed as specified in section 950. Warranty requirements as specified in in Section 957.2(b) do not apply for section 957.2(c)3.

The detection system shall be 100% compatible with the Adaptive Signal System installed and shall have the following functional requirements:

- Sense the presence or passage of vehicles (stop bar and advance detection) by lane.  This can be accomplished using one camera/sensor or multiple cameras/sensors.

- Detect at minimum the detection zones as indicated.

- Built-in external RJ-45 port and power connector.

- If video detection is used, Video output shall be H.264, MJPEG, or MPEG-4

- Minimum video resolution of 720 x 576 streaming on any modern web browser (HTTP).

- Each intersection shall be equipped with at least one camera capable of providing video imagery of the entire intersection that is streamed remotely.  The video shall encompass all approaches at the intersection and provide the user with real-time traffic observations.  This camera may double as a vehicle detection camera. Video output in h.264, MJPEG, or MPEG-4. Minimum video resolution of 720 x 576 streaming on any modern web browser (HTTP).

- Detection sensor shall be in detector mode prior to implementing the adaptive system.

The mounting hardware shall include all clamps, wiring harnesses, and remaining hardware necessary to mount and operate the vehicle detection system according to the manufacturer’s recommendations.  Specific mounting hardware is dependent on the vehicle detection system selected for this project and shall be compatible with both strain pole/span wire and mast arm signal supports.  If recommended by the manufacturer, install luminaire arms on strain pole signal supports to obtain proper detector placement and visibility.  Luminaire arms, if determined necessary, shall be incidental to the vehicle detection system and considered part of the mounting hardware.

  - CONSTRUCTION—

- Adaptive Signal System

As specified in Section 950.3 and as follows:

Request IP addresses for all IP-addressable components of the adaptive system according to the Traffic Signal Remote Communication Policy.

Prior to installation, provide catalog cuts, proof of adaptive system concept detailing the satisfaction of each core functional requirement, and shop testing documentation to the District Traffic Unit for approval.

After installation, the supplier's Engineer shall configure the Adaptive Signal System according to the core functional requirements and for optimal operation of the network considering the objectives as indicated. Traffic flow and anomalous traffic conditions are to be programmed into the Adaptive Signal System. All phase sequencing shall be verified by the municipality and the District Traffic Engineer before final approval.

Before the initial turn-on, verify with the Representative that all systems are working properly. Notify the District Press Office two weeks in advance of the initial system turn-on.

The intersection shall only be removed from current signal operations including time of day plans and detection when the contractor/supplier is actively working at the intersection. The signal shall operate with uninterrupted existing operations during non-working times. Refer to temporary traffic control plans or existing permit plans for operations of existing detection system. Proposed detection may be used to provide uninterrupted service. Interrupted service is defined as disabled detection, communication, or unintended signal operations (signal running free when it should be running time-based coordination). If required, extended uninterrupted service shall be approved by the District Traffic Engineer. The supplier/contractor shall provide duration of interrupted service during the request.

  - Training

The supplier shall provide training to the Department personnel, municipal personnel, municipal consultant engineers and municipal maintenance providers on the setup, operation, and maintenance of the Adaptive Signal System. Training shall be provided for the number of days indicated.

Training shall include the following topics:

- accessing the adaptive signal system remotely to the adaptive signal system through the web interface locally or over an Ethernet network on an authorized computer

- accessing historic data

- testing system failures and identifying problems and applicable troubleshooting techniques (hardware/processor, network/communication, detection failures, and adaptive web interface)

- adjusting and monitoring the adaptive signal system settings

- reporting and analyzing

  - Hardware Adaptive Signal System

As specified in Section 957.3(a) and as follows:

All equipment shall be labeled by the supplier, based on the provided IP addresses, and installed by the Contractor.

Install the Adaptive Signal System in the controller assembly or auxiliary cabinet according to the manufacturer's recommendations and as indicated.

  - Software Adaptive Signal System

As specified in Section 957.3(a) and as follows:

If there is applicable hardware, the supplier shall provide a 5-year warranty, as specified in Section 957.3(a).1.

The contractor and vendor for the Software Adaptive Signal System shall install all applicable components of the Software Adaptive Signal System to be fully operational according to the Adaptive Signal System’s manufacturer requirements.

If applicable, the installation shall include modifying the controller setup and/or programming and/or installing adaptive keys at each intersection, establishing the Central System and configuring the adaptive signal system into the Central System, obtaining and activating the software license for the system or intersections, setup the web-based/adaptive system application/adaptive system interface, and set up the off-site computer/keyboard/monitor, setup with the adaptive signal system manufacturer’s minimum system requirements, and any additional setup requirements.

  - Intersection Adaptive System Detection System

As specified in Section 910.3, 950.3, 954.3, 956.3, and as follows:

Install and mount the vehicle detection system according to the manufacturer’s recommendations to provide minimum area of detection and operation as indicated without detecting conflicting movements. Secure detectors using appropriate mounting brackets according to manufacturer’s recommendations.

Connect the Ethernet cable and power cable according to the manufacturer's recommendations from each vehicle detector back to the controller assembly. Provide repeaters and enclosures if vehicle detectors are over 300 feet from the cabinet.  Provide field validation that the detector equipment is properly working.

All vehicle detection shall be passed through to the controller in the event of adaptive processor failure or a need to disable adaptive operation in the future.  The system shall not operate in “free mode” due to a loss of communication between the controller and the processor unless directed.

The supplier shall provide a copy of any necessary software to the Department, municipality, municipal consultant engineers, and municipal maintenance upon request.

957.4 MEASUREMENT AND PAYMENT—

- Hardware Adaptive Signal System. Each

The price includes all required hardware and configuration for a complete and functional system. 50% of the contract price will be paid upon delivery of equipment. 35% will be paid when all the equipment has been installed, configured, and is a fully functional system. 15% will be paid upon successful completion of the 30-day testing requirements.

All applicable training, 5-year (1827 days) technical, hardware, and software warranty, and spare hardware for all intersections in the adaptive system is included the system cost.

- Software Adaptive Signal System. Each

75% of the contract price will be paid upon setup of software and installation of any hardware. 25% will be paid upon successful completion of the 30-day testing requirements.

All applicable training, 5-year (1827 days) technical, hardware, and software warranty, and software/license fee for all intersections in the adaptive system is included the system cost.

- Intersection Adaptive System Detection System. Each

The price includes controller interface/processor, mounting brackets, conduit, trenching, boring, junction boxes, splicing, luminaire arms, banding, cabling, setting up detection zones, software, and field adjustments for a complete installation at one intersection.

Exposed or buried conduit, trenching, boring, junction boxes, splicing, mounting hardware, and ethernet cable are incidental and shall be provided with the vehicle detection system as indicated and according to the manufacturer’s recommendations.