# Traffic Engineering Software

## General

### Purpose

Various software packages are available for use by traffic engineers.  Traffic analysis software packages are used to optimize traffic flows and capacity, or to simulate traffic flow.  In general, the definition of simulation is the “dynamic representation of some part of the real world achieved by building a computer model and moving it through time.”

The purpose of this chapter is to summarize the Department’s policy on the use of traffic engineering software.

## Specific Software

### Supported Software

The Department supports the following traffic analysis tools, methods, and commercial software packages for analysis of the facility types indicated:

Exhibit 12-1	Supported Traffic Engineering Software

| Tools/Software | Category1 | Facility Type |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Stop Control | Single-Lane Roundabout | Two-Lane Roundabout2 | Signalized Intersection | Urban Streets | Uninterrupted Flow |
| HCM Generalized Service Volume Tables | Sketch-Planning |  |  |  | X | X | X |
| ICU (Intersection Capacity Utilization) | Sketch-Planning |  |  |  | X |  |  |
| FREEVAL-PA | Sketch-Planning |  |  |  |  |  | X |
| Highway Capacity Software | HCM-Based Analytical | X | X | X | X | X | X |
| TRANSYT-7F | Optimization |  |  |  | X | X |  |
| Synchro Version 12 | Optimization and HCM-Based | X3 | X |  | X3 | X3 |  |
| SimTraffic Version 12 | Microscopic Simulation | X |  |  | X | X |  |
| Trip Generation Software | Sketch-Planning | N/A |  |  |  |  |  |
| PTV Vistro | Optimization and HCM-Based | X |  |  | X |  |  |
| TruTraffic | Optimization |  |  |  |  | X |  |
| GDOT Roundabout Analysis Tool | HCM-Based Analytical |  | X | X |  |  |  |

Notes:

1Refer to HCM, Chapter 6, for definitions of Traffic Analysis Tool and Model categories.

2Two lane roundabouts refer to two lanes on any approach or within the circulating roadway.

3HCM 6th Edition reports in Synchro may be accepted since the HCM 7th Edition did not change the methodology.

### Traffic Analysis Tool Selection Process

Based on the recommended traffic analysis tool category or categories identified in Publication 46, Chapter 10.3, the process provided in Chapter 10.3, Traffic Analysis Tool Selection, should be applied to determine which software shown in Exhibit 12-1 should be used when evaluating traffic capacity.

If the desired analytical or simulation software is not found within Publication 46, Chapter 12.2:

1) A written request is to be sent to the appropriate Engineering District Office for consideration;

2) Within the request, the reasons why an alternative analytical or simulation software should be clearly identified along with the added benefits of using the alternative software compared to the Department supported software platforms;

3) The Engineering District Office will evaluate each request and the District Traffic Engineer will provide a written response as to whether the alternative analytical or simulation software could be used on a particular project;

4) Note that an alternative analytical or simulation software analysis should not be submitted to the Department until a written response has been received by the District Traffic Engineer; and

5) If an Engineering District Office receives an alternative analytical or simulation software then coordination with the Bureau of Operations, TSMO Arterials and Planning Section is recommended to ensure that an appropriate way of evaluating the accuracy of the model has been determined.

### Required Use by Department and Consultants

Applicable work done by the Department, or by engineering consultants making submissions for Department review and approval, including but not necessarily limited to design, operational assessments, or Highway Occupancy Permit (HOP) projects, should use the Highway Capacity Manual (7th Edition) and supporting software packages as directed in Publication 46, Chapter 10, and listed in Exhibit 12-1, unless directed otherwise in writing by the Department, and as dictated by a subject project’s scope of work.

Unless an alternative analysis tool is used, level-of-service (LOS) calculations and resultant measures of effectiveness should be calculated using the methodologies established by the Highway Capacity Manual (7th Edition) using a supported HCM-based tool identified in Publication 46, Chapter 12.2.